CS253 Laboratory session 3

Part 1: Evaluating floating point expressions using the maths co-processor.

CODE:

.286

.model medium

.8087

.stack 100h

.DATA

SX      dd 3.0           ;

SY      dd 4.0           ;

cntrl   dw 03FFh         ;

stat    dw 0             ;

INTG    dw 0             ;

CRLF    db 0Dh,0Ah,'$'   ;

.CODE

.STARTUP

FINIT

FLDCW   cntrl

;—— HY = sqrt(SX² + SY²) ——————————————————

FLD     SX

FMUL    ST,ST(0)         ; ST0 = SX*SX

FLD     SY

FMUL    ST,ST(0)         ; ST0 = SY*SY

FADD    ST,ST(1)         ; ST0 = SX²+SY²

FSQRT                    ; ST0 = √(SX²+SY²)

FSTSW   stat

mov     ax,stat

and     al,0BFh

jnz     DONE

FSTP    HY               ;

—

mov     bx,OFFSET HY

add     bx,2

mov     ax,[bx]

mov     bx,ax

mov     cx,16

_P_HIGH:

rol     bx,1

jc      _H1

mov     dl,'0'

jmp     _H2

_H1:

mov     dl,'1'

_H2:

mov     ah,02h

int     21h

loop    _P_HIGH

mov     bx,OFFSET HY

mov     ax,[bx]

mov     bx,ax

mov     cx,16

_P_LOW:

rol     bx,1

jc      _L1

mov     dl,'0'

jmp     _L2

_L1:

mov     dl,'1'

_L2:

mov     ah,02h

int     21h

loop    _P_LOW

mov     dx,OFFSET CRLF

mov     ah,09h

int     21h

————————————

fld     HY

fistp   word ptr INTG    ; INTG ← 5

mov     ax,INTG

xor     dx,dx

mov     bx,10

div     bx

cmp     ax,0

je      _PRINT_REM

add     al,'0'

mov     dl,al

mov     ah,02h

int     21h

_PRINT_REM:

add     dl,'0'

mov     ah,02h

int     21h

mov     dl,'.'

mov     ah,02h

int     21h

mov     dl,'0'

mov     ah,02h

int     21h

DONE:

.EXIT

END

Part 2: So far we have created and compiled a number of assembly language programs using the Microsoft assembler (MASM). Writing a big program using MASM is probably impractical. However, you can embed assembly language in your C\C++ programs. This is something that you could do in practice so as to optimise a piece of code or make use of instructions that are not accessible via the standard libraries (such a MMX, multimedia extension).

This approach has the benefit of allowing you to write the input and output in C/C++ and use the assembly language for high speed calculation.

It should be said that in practice compilers are so good that it is very difficult to write better machine code than they can generate.

CODE

// MASM_FP.cpp : Defines the entry point for the console application. #include "stdafx.h"

#include <stdio.h>

void test(void); // Function prototype (description) int _tmain(int argc, _TCHAR* argv[])

{

test(); return 0;

}

// Put our unmanaged asm code in here void test()

{

unsigned short num1; unsigned short num2; unsigned short result;

printf("Enter first number: "); scanf(" %hd",&num1); printf("Enter second number: "); scanf(" %hd",&num2);

asm

{

}

mov ax, num1 ; Direct addressing to access num1 mov bx, num2 ; put value in num2 into bx

add ax, bx ; ax=ax+bx

mov result,ax; ; put value into result

printf("%hd",result); // Display result

// Wait for enter to be pressed before terminating while(getchar()!=10); // Clear buffer of previous <ret> while(getchar()!=10); // Wait for a new <ret>

}

Part 3: Use the sample code on the next page to generate a suitable frame work for creating code to implement a floating point calculation using Assembly Language from within a C++ program. The program puts the contents of variables on the floating stack and then takes the value on the floating point stack and puts it back into the variable C.

Program 2

// MASM_FP.cpp : Defines the entry point for the console application. #include "stdafx.h"

#include <stdio.h>

void test(void); // Function prototype (description) int _tmain(int argc, _TCHAR* argv[])

{

test(); return 0;

}

void test()

{

float A=2,C=0;

unsigned short cntrl=0x3FF,stat;

asm

{

FINIT

FLDCW cntrl ; Round even, Mask Interrupts FLD A	; Push SX onto FP stack

FSTSW stat	; Load FPU status into [stat]

FSTP C	; Copy result from stack into HY

}

// Binary representation of the 4 bytes, (32 bits) coding HY printf("Binary:");

unsigned char byt; for(int x=3;x>=0;x--)

{

byt=*((unsigned char *)&C+x); for(int y=128;y>0;y/=2)

{

if ((y&byt)==0) printf("0"); else printf("1");

}

}

// Decimal format printf("\nDecimal: %3.0f",C);

// Hex format printf("\nHex:"); for(int x=3;x>=0;x--)

{

byt=*((unsigned char *)&C+x);

printf("%x",(unsigned int)byt);

}

// Decimal 4 byte format printf("\nDecimal (4bytes):"); for(int x=3;x>=0;x--)

{

byt=*((unsigned char *)&C+x); printf("%d,",(unsigned int)byt);

}

// while(getchar()!=10); while(getchar()!=10);

}

Part 4: The aim of the next section is to show how it possible using assembly language to directly access CPU instructions that are not directly available when you use a higher level language.

#include "stdafx.h"

#include <stdio.h>

void test(void); /

int _tmain(int argc, _TCHAR* argv[])

{

test();

return 0;

}

void test()

{

//

union mmx_word {

unsigned char byte[8];

unsigned __int64 value;

};

mmx_word NUM1 = { 0,1,2,3,4,5,6,7 };

mmx_word NUM2 = { 1,1,1,1,1,1,1,1 };

__asm {

//

movq mm0, NUM1

movq mm1, NUM2

paddb mm0, mm1

movq NUM1, mm0

emms

}

printf("Result bytes: ")

for (int i = 0; i < 8; i++) {

printf("%u", (unsigned int)NUM1.byte[i]);

if (i < 7) printf(",");

}

printf("\n");

/

while (getchar() != '\n');

while (getchar() != '\n');

}