Task1

Task2

.MODEL small

.STACK

.DATA

; You can define your initials here if you don't want to hardcode them in the code

; INITIAL1 DB 'S'

; INITIAL2 DB 'J'

.CODE

.STARTUP

mov ax,0b800h     ; Set the video memory segment address to ax

mov es,ax         ; Assign the value of ax to the es register, es now points to the video memory segment

; Calculate the offset address for the target position (row 10, column 12)

; Row number y = 10, Column number x = 12

; Offset = (y * 160) + (x * 2) = (10 * 160) + (12 * 2) = 1600 + 24 = 1624 (decimal) = 658h

mov bx, 1624      ; Store the calculated offset address in bx

; --- First Initial ---

; Character: Your first initial (e.g., 'S')

; Color: Yellow on black background (attribute 0Eh)

mov al, 'C'       ; Put your first initial into the al register (please replace 'S')

mov es:[bx], al   ; Write the character in al to the current video memory location (es:bx)

inc bx            ; bx points to the attribute byte location of the first character

mov al, 0Eh       ; Set attribute to yellow on black (yellow foreground E, black background 0)

mov es:[bx], al   ; Write the attribute byte to video memory

inc bx            ; bx points to the ASCII code location of the second character

; --- Second Initial ---

; Character: Your second initial (e.g., 'J')

; Color: Bright blue on yellow background (attribute 69h)

mov al, 'J'       ; Put your second initial into the al register (please replace 'J')

mov es:[bx], al   ; Write the character in al to the current video memory location (es:bx)

inc bx            ; bx points to the attribute byte location of the second character

mov al, 69h       ; Set attribute to bright blue on yellow (bright blue foreground 9, yellow background 6)

mov es:[bx], al   ; Write the attribute byte to video memory

quit:

.EXIT             ; End of program

END

Task3

3.1 k1.asm

* k1.asm Detects keypress from the keyboard      */

;/* If the key presses is 'h' or'H'                 */

;/* the routine prints 'Hello' on the screen       */

; By  C.Markham

; Revised  March 2016

.MODEL    small

.STACK

.DATA      ; DS will may change when TSR operates

.CODE

.STARTUP

push    es              ; Save existing value of ES.

mov     ax, 0000h       ; Set ES (extra segment) to page 0

mov     es, ax          ; Note page 0 is vector address table

mov     bx, 024h        ; Vector address for keyboard INT 9, IRQ1

mov     ax, es:[bx]     ; ax=[0000:0020], CS to jump to at end of our new ISR

mov     vtabip, ax      ; Store the CS, 1st two bytes of table

inc     bx

inc     bx

mov     ax, es:[bx]     ; ax=[0000:0020], IP to jump to

mov     vtabcs, ax      ; Store the IP, 2nd two bytes

pop     es              ; Restore the value of es

push    es              ; Change vector address to ISR routine

mov     ax, 0000h       ; Vector address table segment

mov     es, ax

mov     bx, 024h

mov     ax, OFFSET isr  ; Set IP of vector table to new ISR

mov     es:[bx], ax

inc     bx

inc     bx

mov     ax, cs          ; The ISR is in the same segment as the program.

mov     es:[bx], ax     ; New CS in vector table is the code segment of

pop     es              ; this program.

mov     ah, 031h        ; TSR Entry code ah=31h,al=0h

mov     al, 00h

mov     dx, 1024        ; DX=no of bytes to remain resident

int     021h            ; Terminate and stay resident.

jmp     quit            ; Historic code, never reached

isr:      push    ax              ; Start of code you wish to call ever 50mS (18ms PC).

push    bx              ; Interrupts don’t store general registers, you must.

push    cx

push    dx

push    di

push    si

push    bp

push    es

push    ds

mov     dx, 60h         ; Read the last scan code

in      al, dx

and     al, 127         ; Mask MSB 1=Key Down, 0=Key Up remove if just up or down

cmp     al, 23h         ; Was a h/H pressed on the XT keyboard

jnz     ovr1

; Change to

; <space> scan

; code rather

; than ‘h’.

; Remove the

; ‘and al,127’

mov     ax, 0b800h      ; Base address of screen memory

mov     es, ax

mov     bx, 140         ; Screen position (1,70)

mov     al, 'H'         ; Print 'Hello' on the screen

mov     es:[bx], al

inc     bx

inc     bx

mov     al, 'e'

mov     es:[bx], al

inc     bx

inc     bx

mov     al, 'l'

mov     es:[bx], al

inc     bx

inc     bx

mov     al, 'l'

mov     es:[bx], al

inc     bx

inc     bx

mov     al, 'o'

mov     es:[bx], al

jmp     ovr2

ovr1:     mov     ax, 0b800h      ; If not an H overwrite top right with ' '

mov     es, ax

mov     bx, 140         ; Screen position (1,70)

mov     cx, 5

again:    mov     al, ' '

mov     es:[bx], al

inc     bx

inc     bx

loop    again

ovr2:     mov     dx, 020h        ; Non Specific end of interrupt command

mov     al, 020h        ; to sent 8259 #1 controller.

out     dx, al

pop     ds              ; Restore all registers previously save on

pop     es              ; the stack.

pop     bp

pop     si

pop     di

pop     dx

pop     cx

pop     bx

pop     ax

db      0EAh            ; jump far  (instruction created on the fly...)

vtabip    dw      0               ; IP offset

vtabcs    dw      0               ; CS segment

quit:     .EXIT                   ; Historic code, never reached

END

3.2 t1.asm

.MODEL    small

.STACK

.DATA

.CODE

.STARTUP

push    es

mov     ax, 0000h

mov     es, ax

mov     bx, 024h        ; INT 9 vector offset

mov     ax, es:[bx]     ; Old IP

mov     cs:[OFFSET vtabip_label], ax

inc     bx

inc     bx

mov     ax, es:[bx]     ; Old CS

mov     cs:[OFFSET vtabcs_label], ax ;

pop     es

mov     ah, 031h

mov     al, 00h

mov     dx, OFFSET EndResidentPart ;

sub     dx, OFFSET isr             ;

shr     dx, 4

inc     dx

int     021h

jmp     quit            ;

isr:

;

push    cs

pop     ds              ;

db      0EAh            ;

vtabcs_label dw   0               ;

quit:

EndResidentPart LABEL BYTE ;

.EXIT

END

3.3 Kt1

.MODEL small

.STACK 100h

.DATA

; No data section for TSR

.CODE

ORG 100h        ; COM file format

;-------------------------------------------------------------------------------

; Data definitions (stored in code segment for TSR)

;-------------------------------------------------------------------------------

CounterValue  db '0','0','0','0'    ; Counter ASCII characters

OldKeyIP      dw ?                  ; Save old keyboard ISR IP

OldKeyCS      dw ?                  ; Save old keyboard ISR CS

MyProg PROC FAR

; Initialize

push cs

pop ds              ; DS = CS for COM program

; --- Save old keyboard interrupt vector (INT 9) ---

push es

xor ax, ax

mov es, ax          ; ES = 0000h

mov bx, 9 * 4       ; INT 9 vector address

mov ax, es:[bx]

mov cs:[OldKeyIP], ax

mov ax, es:[bx+2]

mov cs:[OldKeyCS], ax

pop es

; --- Set new keyboard interrupt vector ---

cli                 ; Disable interrupts

push es

xor ax, ax

mov es, ax

mov bx, 9 * 4

mov ax, OFFSET KeyISR

mov es:[bx], ax

mov ax, cs

mov es:[bx+2], ax

pop es

sti                 ; Enable interrupts

; --- Initialize counter display ---

call DisplayCounter

; --- TSR exit, stay resident ---

mov dx, OFFSET EndResidentPart

add dx, 15          ; Round up to paragraph boundary

shr dx, 4           ; Convert to paragraphs

mov ax, 3100h       ; TSR function, return code 0

int 21h

MyProg ENDP

;-------------------------------------------------------------------------------

; Keyboard Interrupt Service Routine (ISR)

;-------------------------------------------------------------------------------

KeyISR PROC FAR

push ax

push bx

push cx

push dx

push si

push di

push ds

push es

push bp

; Set DS to code segment

push cs

pop ds

; Read keyboard scan code

in al, 60h

; Check if Enter key (scan code 1Ch)

cmp al, 1Ch

je IsEnterKey

; Check if Space key (scan code 39h)

cmp al, 39h

je IsSpaceKey

; Not our key, chain to original handler

jmp ChainToOriginal

IsSpaceKey:

; Increment counter

mov cx, 4

mov si, OFFSET CounterValue + 3    ; Start from least significant digit

IncrementLoop:

mov al, [si]

inc al

cmp al, '9' + 1

jne StoreDigit

mov al, '0'                        ; Carry, set current digit to 0

mov [si], al

dec si

loop IncrementLoop

jmp UpdateDone

StoreDigit:

mov [si], al

UpdateDone:

call DisplayCounter

jmp SendEOI

IsEnterKey:

; Reset counter to 0

mov byte ptr [CounterValue],   '0'

mov byte ptr [CounterValue+1], '0'

mov byte ptr [CounterValue+2], '0'

mov byte ptr [CounterValue+3], '0'

call DisplayCounter

SendEOI:

; Send EOI to 8259A interrupt controller

mov al, 20h

out 20h, al

; Restore registers and return

pop bp

pop es

pop ds

pop di

pop si

pop dx

pop cx

pop bx

pop ax

iret                                ; Interrupt return

ChainToOriginal:

; Restore registers

pop bp

pop es

pop ds

pop di

pop si

pop dx

pop cx

pop bx

pop ax

; Jump to original keyboard handler

jmp dword ptr cs:[OldKeyIP]

KeyISR ENDP

;-------------------------------------------------------------------------------

; Display counter subroutine

;-------------------------------------------------------------------------------

DisplayCounter PROC NEAR

push ax

push bx

push cx

push dx

push si

push ds

push es

; Set DS to code segment

push cs

pop ds

; Set ES to video memory

mov ax, 0B800h

mov es, ax

; Screen position (row 0, column 0)

mov bx, 0                           ; (0 * 80 + 0) * 2

; Display 4 digits

mov cx, 4

mov si, OFFSET CounterValue

DisplayLoop:

mov al, [si]                        ; Get character

mov es:[bx], al                     ; Write character

inc bx

mov byte ptr es:[bx], 07h           ; White on black attribute

inc bx

inc si

loop DisplayLoop

pop es

pop ds

pop si

pop dx

pop cx

pop bx

pop ax

ret

DisplayCounter ENDP

EndResidentPart LABEL BYTE              ; TSR end marker

END MyProg