Task1

void setup() {

// initialize digital pin LED_BUILTIN as an output.

pinMode(LED_BUILTIN, OUTPUT);

}

// the loop function runs over and over again forever

void loop() {

digitalWrite(LED_BUILTIN, HIGH);  // turn the LED on (HIGH is the voltage level)

delay(1000);                      // wait for a second

digitalWrite(LED_BUILTIN, LOW);   // turn the LED off by making the voltage LOW

delay(1000);                      // wait for a second

}

Task2

// 定义变量来追踪按钮和LED的状态

int newState = HIGH, oldState = HIGH, ledState = LOW;

void setup() {

pinMode(3, OUTPUT); // 设置引脚3（LED）为输出模式

pinMode(2, INPUT);  // 设置引脚2（按钮）为输入模式

}

void loop() {

// 读取按钮引脚的当前状态

newState = digitalRead(2);

// 检查按钮是否刚刚被按下（从高电平变为低电平）

if ((oldState == HIGH) && (newState == LOW)) {

// 短暂延迟以消除“接触点抖动”

delay(20);

// 检查LED当前的状态

if (ledState == HIGH) {

// 如果LED是亮的，就把它熄灭

digitalWrite(3, LOW);

ledState = LOW; // 更新LED状态变量

} else {

// 如果LED是灭的，就把它点亮

digitalWrite(3, HIGH);

ledState = HIGH; // 更新LED状态变量

}

}

// 更新旧状态，为下一次循环做准备

oldState = newState;

}

Task3

int sensorValue;

void setup() {

Serial.begin(9600);

}

void loop() {

sensorValue = analogRead(A0);

Serial.println(sensorValue);

delay(10);

}

Task4

#include <LiquidCrystal.h>

int sensorValue;

int wiperPosition;

// LiquidCrystal lcd(Reset, Enable, D4, D5, D6, D7)

LiquidCrystal lcd(7, 10, 8, 11, 9, 12);

void setup() {

lcd.begin(16, 2);

}

void loop() {

lcd.setCursor(0, 0);

sensorValue = analogRead(A0);

wiperPosition = map(sensorValue, 0, 1023, 0, 270);

// 在 LCD 上打印文本和变量

lcd.print("Pos: ");

lcd.print(wiperPosition);

lcd.print("   ");

delay(10);

}